Putt Skee

Operation of Electronics

Once the machine is all set up and all cables are plugged in, Plug in your main power cable.  Remember, there is a short black cable with a barrel plug at the end of it coming from the scoring drain on the back of the drain panel.  This plugs into the bottom of the playfield on the left side (looking at the machine from the back).

When the game boots up, it will flash “PuttSkee” 4 times in Red and Blue.  After which it will sequence through several screens telling you to come play and press start.  On your putter stand lives the control panel.  There are three buttons; Start, Reset, and Set.  Start will start the game at any point in the idle mode of the game.  It will also start the game from the Settings menu.  The Set button will take you to a settings menu where you can select the two game modes.  Once in the menu, use the Set button to cycle through the different timer durations from 15 seconds to 60 seconds.  The last setting is 9 ball mode.  At any time in the settings menu, you can press the start button and the game will start at the setting that was previously on the display board.  If the game is left inside the settings menu for more than 5 seconds, the previous menu item that was displayed will be locked in and set!

Timer Mode:

When in timer game mode, the Timer Play graphic will be illuminated on the score board and the game will start at the time that was selected.

9 Ball Mode:

When in 9 Ball game mode, you will have 9 balls to make as many points as you can with 9 shots like traditional Skee Ball.  The 9 Ball Play graphic will be illuminated on the score board.

The reset button will, at any time during game play, exit you back to the idle mode and your score will be lost.  Once the first game is played, when the game is plugged in, the new high score idle screen will appear with the score obtained in that game.  If a score of 0 points was made, no high score will be displayed and the game will go back to the idle screens.  Every time a game is played, if the score is greater than the previously made high score, the new one will be displayed.  To reset the high score, simply unplug the game for 10 seconds and plug it back in.

Once a game is finished, if you have made at least 10 points, the score board will display “Game Over” and it will flash your score 4 times and return you to the idle screens.  Again if you have made a new high score, the score board will display “New High Score” then flash your score 4 times and this new score will show up in the idle screens.

Troubleshooting:

If for any reason the program locks up (freezes, doesn’t respond to controls, scoring any balls, jumbled garbage on display) simply unplug the game for 10 seconds and plug it back in.

If your controls do not work, please check to see if the connectors at both ends of the cable are fully seated into the sockets correctly.  Remember these connectors will only connect one way around.  This cable harness is bi-directional so it can be plugged in at both ends, the connectors are the same at either end.

If nothing shows up on the score board, again please make sure that the connection is seated all the way into the socket.

If the Playfield lights are not on, make sure you have the short black cable with the barrel connector plugged into the playfield.

If balls are getting stuck inside the drain, the pipes at the bottom of the machine may need to be adjusted.  Do not remove any of the screws in the pipes!!  Simply flex the pipes so they are at a down hill slope so the ball(s) drain.

This is a one ball per turn game.  DO NOT attempt to Putt or throw more than one ball at the same time down the play field.  This can and will result in jammed up balls.

If a ball(s) are getting stuck before the drain, please make sure playfield rings are close to flush with the front of the Drain Panel.  If the stuck ball still does not drain, attempt to put another ball down the drain to see if that will get it un-stuck.

Pipes can be taken apart with a Philips head screwdriver.  Make sure when re-installing the screws that you leave them sticking out about ⅛ of an inch.  This is to prevent the screws from penetrating all the way through the inner pipe and causing jams.  Also make sure that they get put back together exactly how them came apart.  Failure to do this will also result in jams.

Technical Information:

The entire machine has one power supply installed for everything.  This power supply is rated for 24VDC @ 20 Amps.  If you wanted to add more LED strips, or any other stuff.

Inside the Score board is where all the “brains” of the operations are installed.  There is also a step down from 24V to 5V inside the score board to power all the lower level electronics.  The cable at the back of the score board contains 24VDC from the power supply and all the logic sensors/switches.  There is not a 5V rail on this connector!

If you need a 5V rail for something that you want to install on the machine, please add a buck converter of your own on to the 24V power supply already installed into the machine.

Notes:

The modifications of this machine are the sole responsibility of the purchasing party ONLY and will not be covered by any warranties herein that were discussed upon purchasing this machine.